[image: image1.jpg]


[image: image2.wmf] 

[image: image3.jpg]


 
12 MegapixelCam
This advanced webcam features 12 megapixel HD images and live streaming video in one construction camera system.
[image: image4.jpg]


[image: image5.wmf] 



      
EarthCam 12 MegapixelCam Specification                                                    A&E Short Form*

1. Physical
Dimensions: 10.55” W x 6.19” H x 26.5” L (26.79cm W x 15.72cm H x 67.31cm L)

Weight: 16 lbs (7.26 kg)

2. Environmental
Operational Temperature: -10ºF to + 120ºF (-23ºC to +49ºC)

Thermostatically Controlled Heater and Blower

24/7 Operation in All Weather Conditions

3. Construction
Powder Coated Aluminum Housing with Stainless Steel Fittings for Padlocks

Impact Resistant Viewing Window, 0.25” (6.35mm) Thick

Two UL Rated Compression Glands, Gas Spring Lid, Adjustable Camera Sled

4. Camera 
Imager: 12.2 Megapixels (4272 x 2848 Pixels), 22.2mm x 14.8mm CMOS Image Sensor 
Lens: F/3.5-5.6, 18mm - 55mm, 3x Optical

Auto Features:  ISO, Shutter, White Balance and Focus

Video Compression: Motion JPEG, Preview: 528 x 352 Streaming Video Resolution

Communication: 10base-T/100base-TX Ethernet, IP Addressing: Dynamic or Static

Designed For “EarthCam Control Center - Megapixel Edition – Fixed with Mobile Support” Software

5. Electrical

Operational Voltage: 120VAC (230VAC Available). Power Consumption: 130W

6. Interface and Online Software Features

Software Delivered by Vendor as a Managed Service

Displays Company Logo and Project Name

Capable of Viewing Actual Live Video
HD Snapshot on Demand
Picture in Picture to View Live Video, While Viewing High Definition Images

Calendar Based Navigation System for Selecting Specific Images

Multifunction Image Browsing 

Pan, Tilt and Zoom Capability within a High Definition Image

A Multiview Screen to View All Cameras on Project at the Same Time

Graphical Mark-Up Tools for Detailing and Creating Overlays on Images

Graphical Weather Applet Displaying Ten Points of Local Weather Data and 48-hour Forecast 

Remote Solar Monitoring Screen Displaying the DC Amperage Output of Solar Panels

Remote Battery Monitoring Screen Displaying Battery Voltage, Temperature and Status

Remote Cellular Monitoring Screen Displaying Connectivity, Network Traffic and Modem Temperature

Remote Wireless Radio Monitoring Screen Displaying Connectivity, Network Traffic and 

Google Map Features Including Wireless Radio Locations

Share Image Tools: Save, Print, Email and Post to Message board or Mobile Devices

Automated Progress Report in PowerPoint, Open Office and PDF Formats

Map, Aerial and Satellite View by Google

Time-Lapse Feature Includes – Instant Time-Lapse Play Back by Day, Week, Month or Year

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Security Features Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password

All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor

Product Features





HD images/Live Video in one system.





· 12 Megapixel HD fixed position camera


· High Definition images: 4272 x 2848 pixels


· 528 x 352 live streaming video preview


· Digital presets and Pan/Tilt/Zoom within captured image


· Take and share on demand snapshot of the project remotely


· Heavy-duty enclosure with heater and fan 


· Unique double locking mount for jitter-free documentation


· Enhanced self-healing technology


· Superior quality time-lapse playback


· Fast, dependable, solid state Linux operating system


· Available in Wide Angle and Skyscraper versions





�





� EMBED Word.Picture.8  ���





�














�





Architectural & Engineering Specifications


Product:  ECCS03135


Revised:  09-12-2012





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/12megapixelcam" �http://www.earthcam.net/aespecs/12megapixelcam�





For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems�








*Visit EarthCam.net for the full version of this A&E specification.                       9/12 ©2012 EarthCam, Inc.
84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net

_1301836141.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







