EarthCam Webcam Technology                                                                                                SecurityCam Lite 
[image: image2.jpg]e

EarthCam



 A&E Specifications                                                                                                                       Robotic System


EARTHCAM SECURITYCAM LITE

PART 2- PRODUCT 

2.1     MANUFACTURER

   A.
EarthCam, Inc.

800.EarthCam (327.8422) toll free

201.488.1111voice

201.488.1119 fax

84 Kennedy Street,

Hackensack, NJ 07601.

www.EarthCam.net 

[image: image1]
2.2
SYSTEM REQUIRMENTS

A. The indoor/outdoor camera system shall consist of an IP66 weather resistant aluminum and plastic 
housing with an acrylic optically clear dome to be mounted on a fixed pole, wall, parapet or ceiling.

B. The camera shall provide day/night functionality, maintaining high image quality even in low light 
conditions, including H.264 and Motion JPEG streams at full frame rate in 4CIF resolution.

C. The camera shall upload both images and video over a wireless cellular modem, a wireless point-to-
point system or a RJ-45 hardwired connection to a DSL or cable modem.

D. The content shall be sent to a secure, password protected website with an interface and online 
software features provided by the vendor as a managed service.

E. The system shall operate on 120VAC (230VAC  available) and have a maximum power consumption of 
15.6W using Power Over Ethernet(IEEE 802.3af Class 3).

2.3 EQUIPMENT

A. Robotic Camera: Fast and smooth P/T/Z motion with 360º rotation. Live streaming video and lens assembly consisting of a charge coupled device (CCD) camera with a remotely controlled focal lens with the following features:

1. Imager: 1/4” CCD.

2. Resolution: 704 x 480

3. Frame rate: H.264 up to 30/25 fps in all resolutions. Motion JPEG: up to 30/25 fps in all 
resolutions.

4. Sensitivity: Color 1 lux at 30 IRE F1.6, B/W 0.3 lux at 30IRE F1.6.

5. Lens: 3.8mm – 46mm, Zoom: 12x optical, 4x digital.

6. Auto features: Focus, ISO, iris, shutter, white balance and day/night. 

7. F-Stop: F/1.6 – 2.7 

8. Pan range: 360º continuous at 1.8 to 100º/sec.

9. Tilt range: +2º to -92º at 1.8º a to 100º/sec.

10. 100 preset positions.

B. All weather enclosure:  Aluminum and plastic housing and acrylic dome.

1. Nema 4x molded fiberglass polyester.

2. Dimensions: 4.7” (119mm) Dome, 6.3” (159mm) overall length by 7.4” (188mm) diameter.

3. Weight (with camera): 2.6lbs (1.2Kg)

4. Built in thermostatically controlled heater.

5. Operating temperature: -4ºF to 122ºF (-20ºC to 50ºC).

6. Corrosion resistant stainless steel hardware, back plate and locking latches. 

7. Brass insert molded and mounting tabs.

C.     
Equipment enclosure: Compression-molded fiberglass enclosure with with quick-release latches (padlock provisions included in each latch)

1. Dimensions 10.00” H x 8.00” W x 6.00” D (254mm x 203mm x 152mm)

2. Cable entry: UL rated compression glands.

3. Operational voltage: 120VAC (230VAC Available) Output voltage: 24VAC.

2.4 INTERFACE AND ONLINE SOFTWARE 

A. Remote Access: Contractor’s system vendor shall provide an internet based interface and online software as a managed service, to allow the viewing of all high-definition digital still images captured and stored from any location with internet access via a secure password protected website.

1. Maintain images on the system vendor’s website for reference available at all times during the  
life of the project and for not less than 60 days after completion.

2. All images will be protected on servers owned and operated by the system vendor and located 
in secure area at the system vendor’s location.

B. Online interface features shall be:

1. Software delivered by vendor as a managed service.

2. Displays company logo and project name.

3. Capable of viewing live video.

4. Robotic Pan, Tilt, Zoom control of camera system.

5. Calendar based navigation system for selecting specific images.

6. Multifunction image browsing.

7. A multiview screen to view all of the cameras on a project at the same time.

8. Graphical mark-up tools for detailing and creating overlays on images.

9. Graphical weather applet displaying ten points of local weather data.

10. Remote solar monitoring screen displaying the DC amperage output of solar panels.

11. Remote battery monitoring screen displaying battery voltage, temperature and status.

12. Remote cellular monitoring screen displaying connectivity, network traffic and modem 
temperature.

13. Share image tools: save, print, email and post to message board or mobile devices.

14. Automated progress reports in Power Point, Open Office and PDF formats.

15. Aerial and satellite photography library. 

16. Time lapse features include – Instant time lapse play back by day, week, month or year.

17. Machine to machine self-healing technology that automates maintenance of camera up to 288 
times daily.

18. Account security features include – Four levels of password protection, IP address block/ 
permission and SSL protection of the user login password.

19. All images are the copyright of the client and protected on secure servers owned and operated 
by the system vendor.

PART 3 -  EXECUTION

3.1 preparatioN

A.
Unpack camera system components and save packing materials (box and foam) for future shipment of camera system including associated appurtenances and mounting equipment to Owner or Manufacturer as required.

3.2 INSTALLATION

   A. 
General:

1.
Install camera system in accordance with manufacturer's printed instructions, State and Municipality codes and requirements and approved submittals.

2.  Install units plumb and level and at proper angle to provide maximum field of view of on-site operations.

3.   Install camera so that the sun or any man made light sources will not be in the direct field of view of the camera at any time during operation.

4.
Install camera at elevation that will provide uncompromised visual coverage of desired area.

5.
Securely and rigidly fasten products in place.

4.   Connect camera to power.    

3.3 FIELD QUALITY CONTROL

A.      Pre-installation Testing: Test camera on-site at ground level prior to mounting unit in its intended   elevated position.

1.  Contact the system vendor not less than 24 hours in advance of installation for testing.

2.  Connect unit.

3.  After 30 minutes, contact the system vendor and require system vendor to remotely confirm camera is operating properly.

4.  Install cameras in approved locations.

3.4    CLEANING

             1.  Clean installed items using methods and materials recommended in writing by manufacturer.

   
 2. Clean camera system components, including camera-housing windows, lenses, and monitor screens.

3.5     INSTRUCTION

A.      Engage a factory-authorized service representative by phone to instruct contractor’s personnel in    procedures to adjust and maintain camera equipment.

1.   Instruct personnel on procedures and schedules for troubleshooting and maintaining equipment.

2.   Explain methods of determining optimum alignment and adjustment of components.

3.6     OPERATION, TERMINATION, AND REMOVAL

A.
Maintenance: Maintain camera equipment in good operating condition on a 24-hour basis until removal.

   B.      Termination and Removal: Removal of camera system when instructed by the owner. 

1.   Camera system including associated appurtenances and mounting equipment are property of Owner.

END OF SECTION 

© 2012 EarthCam, Inc.
Page 3 of 3
05.15.2012


[image: image2.jpg]