[image: image1.jpg]


[image: image2.wmf] 

[image: image3.png]


 
SecurityCam Lite

This powerful network security camera is an outdoor ready, day and night pan/tilt/zoom camera designed for easy and cost-efficient installation.
[image: image4.jpg]


[image: image5.wmf] 



       


EarthCam SecurityCam Lite Specification   
                                            A&E Short Form*

1. Physical
Camera Enclosure Dimensions: 4.7” (119mm) Dome, 6.3” (159mm) overall length by 7.4” (188mm) diameter.
Camera Enclosure Weight (with Camera): 2.6lbs (1.2Kg)

Equipment Enclosure Dimensions: 10.00” (254mm) H x 8.00”(203mm) W x 6.00”(152mm) W

2. Environmental

Operating Temperature: -4ºF to 122ºF (-20ºC to 50ºC)

Thermostatically Controlled Heater and Defroster

3. Construction
   The indoor/outdoor camera system consists of an IP66 weather resistant aluminum and plastic housing
   with an acrylic optically clear dome to be mounted on a fixed pole, wall, parapet or ceiling.  

4. Camera 

Image Sensor:  1/4” CCD

Resolution: 704 x 480

Frame rate: H.264 up to 30/25 fps in all resolutions. Motion JPEG: up to 30/25 fps in all resolutions.

Sensitivity: Color 1 lux at 30 IRE F1.6, B/W 0.3 lux at 30IRE F1.6.

Lens: 3.8mm – 46mm, Zoom: 12x optical, 4x digital.

Auto features: Focus, ISO, iris, shutter, white balance and day/night. 

F-Stop: F/1.6 – 2.7 100 preset positions.

Operator Controlled Movement for Pan/Tilt Functionality, IP66-rated Protection, Easily Panned and Tilted by Operator

Pan range: 360º continuous at 1.8 to 100º/sec. Tilt range: +2º to -92º at 1.8º a to 100º/sec.

Designed for the EarthCam Control Center Software

5. Electrical
Operational Voltage: 120VAC (230VAC Available) , Maximum power consumption: 15.6W
 (using Power Over Ethernet(IEEE 802.3af Class 3))

6. Interface and Online Software Features
Software Delivered by Vendor as a Managed Service

Displays Company Logo and Project Name

Capable of Viewing Actual Live Video
Robotic Pan, Tilt, Zoom Control of Camera System
Picture in Picture to View Live Video, While Viewing High Definition Images

Calendar Based Navigation System for Selecting Specific Images

Multifunction Image Browsing 

Pan, Tilt and Zoom Capability within a High Definition Image

A Multiview Screen to View All Cameras on Project at the Same Time

Graphical Mark-Up Tools for Detailing and Creating Overlays on Images

Graphical Weather Applet Displaying Ten Points of Local Weather Data and 48-hour Forecast 

Remote Solar Monitoring Screen Displaying the DC Amperage Output of Solar Panels

Remote Battery Monitoring Screen Displaying Battery Voltage, Temperature and Status

Remote Cellular Monitoring Screen Displaying Connectivity, Network Traffic and Modem Temperature

Remote Wireless Radio Monitoring Screen Displaying Connectivity, Network Traffic and 

Google Map Features Including Wireless Radio Locations

Share Image Tools: Save, Print, Email and Post to Message board or Mobile Devices

Automated Progress Report in PowerPoint, Open Office and PDF Formats

Map, Aerial and Satellite View by Google

Time-Lapse Feature Includes – Instant Time-Lapse Play Back by Day, Week, Month or Year

Machine to Machine Self-Healing Technology Automates Maintenance of Camera Up to 288 Times Daily

Account Security Features Include – Four Levels of Password Protection, IP Address Block/ Permission and SSL Protection of User Login Password
All Images are the Copyright of the Client and Protected on Secure Servers Owned and Operated by the System Vendor
Product Features





Ideal for remote monitoring and surveillance.





· Remote pan/tilt/zoom control with multiple preset compositions�· 12x optical, 4x digital zoom�· 360° pan with auto-flip�· Color by day and black-and-white infrared vision at night�· Motion detection with alarm trigger�· Simultaneous MPEG-4 and Motion JPEG�· Two way audio support�· Fast, dependable, solid state Linux system�· Outdoor and indoor environments


· Fully assembled


· Easy to install








�





� EMBED Word.Picture.8  ���





�











�





Architectural & Engineering Specifications


Product:  ECCS04117


 Revised:  05-22-2012





DOWNLOAD ELECTRONIC VERSION OF THIS SPECS





� HYPERLINK "http://www.earthcam.net/aespecs/securitycamlite" �http://www.earthcam.net/aespecs/securitycamlite�





For additional EarthCam A&E specs visit: � HYPERLINK "http://www.earthcam.net/aespecs/systems" ��http://www.earthcam.net/aespecs/systems�








*Visit EarthCam.net for the full version of this A&E specification.                           05/22 ©2012 EarthCam, Inc.

84 Kennedy Street,  Hackensack,   NJ  07601  USA      Ph.  201.488.1111     Fax.  201.488.1119     info@earthcam.net

_1310282618.doc
[image: image1.jpg]Q EarthCam

== Webcam Technology







