EarthCam, Inc.

800.EarthCam (327.8422) toll free

201.488.1111 voice

201.488.1119 fax

84 Kennedy Street

Hackensack, NJ 07601

www.EarthCam.net

GENERAL NOTES TO SPECIFIER:

This Specification Section has been prepared as an aid to the design professional who is preparing a project master specification.

All care has been extended to make sure this document follows the standards set by CSI MasterFormat, 2004 edition.

This document should be edited to properly suit your particular projects requirements.

You must carefully modify and delete items that are not applicable.

You must verify that referenced Section titles are correct.

This Section assumes the Project Manual will contain complete Division 1 documents.

This is written as a closed proprietary specification. At the time of preparation no other System Vendors were known to have a comparable system.

Revise header and footer to suit project requirements.

Electronic versions of this specification are based on Microsoft Word style formatting.

When editing is complete, delete all text on this page, then remove the section break at the top of the next page to remove this page from the document.

section 01 32 34.01  – Earthcam megapixelcam AdVANCED system

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes an integrated, professional-grade, high resolution digital webcam system designed specifically for the construction industry as a turnkey package including camera(s) and related hardware, mounting equipment, software, wireless cellular data transmission service, website hosting, image hosting and storage, online interface for the system and technical support.

        - Related Sections:

          - Division 1 Section "Photographic Documentation" for periodic construction photographs.

          - Division 1 Section "Closeout Procedures" for submitting digital photographs as Project Record Documents at Project closeout.

          - Division 16 Sections for electrical service and connections, as needed.

      - DEFINITIONS

        - CCD: Charge-coupled device.

        - System Vendor (EarthCam, Inc.): Provider of camera system hardware and software and host maintaining off-site server, data storage devices, and troubleshooting software and equipment. Contractor shall maintain an active contract for System Service for duration of Contract Time unless other term is agreed upon in writing by the Owner. Cost for System Service shall be included in the Contract Sum.

        - System Service: Host services provided by System Vendor (EarthCam, Inc.) including image acquisition, transfer, backup, periodic upgrades to the system, viewing access via a maintained interface on the Internet and on-line storage of images for duration of the Service Contract.

      - SUBMITTALS

        - Project Drawings:

          - Site Plan: Submit site plan of project site and building making sure to note North, possible vantage points for the cameras and Owner’s requirements and restrictions.

          - Building Drawings: Submit drawings of the building showing over all dimensions and a rendering reflecting the finished building.

        - Quality Assurance Submittals:

          - Manufacturer's Instructions: Follow manufacturer’s installation and testing instructions.

        - Project Completion:

          - Digital Images: Submit digital still images exactly as originally recorded in the digital camera, without alteration, manipulation, editing, or modifications using image-editing software.

            - Date and Time: Include date and time in filename for each image.

            - Format: Submit an identifiable directory listing archive of all digital still images on an external hard drive or DVD format.

          - Time-Lapse Movie: Compile select digital still images into a time-lapse movie of the construction period. Optimize images included and run-time length of movie to suit Owner's requirements.

      - QUALITY ASSURANCE

        - Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

          - Factory assemble camera system from components bearing UL Classification Marking indicating that materials have been produced under UL's Classification and Follow-Up Service.

        - Comply with NECA 1, "Standard Practices for Good Workmanship in Electrical Construction."

        - Comply with NFPA 70, "National Electrical Code."

        - Manufacturer Qualifications: Company specializing in manufacturing Products specified in this Section with minimum 12 years documented experience.

      - DELIVERY, STORAGE, AND HANDLING

        - Deliver materials in original EarthCam packages and containers with seals unbroken and bearing manufacturer's labels.

        - Store materials to comply with EarthCam's directions to prevent deterioration from moisture, heat, cold, direct sunlight, or other causes.

      - PROJECT CONDITIONS

        - Environmental Conditions: Capable of withstanding the following environmental conditions without mechanical or electrical damage or degradation of operating capability:

          - Exterior Environment: System components installed in locations exposed to weather shall be rated for continuous operation in ambient temperatures of minus 10º F to plus 120º F dry bulb and 20 to 90 percent relative humidity, condensing. Rate for continuous operation when exposed to rain as specified in NEMA 250, winds up to 85 mph. NEMA 250, Type 3R enclosures.

      - COORDINATION

        - Coordinate layout and installation of cameras to avoid interference from trees and to prevent sunlight and light from fixtures entering directly into the camera lens.

      - WARRANTY

- Warranty: Manufacturer’s standard form in which manufacturer agrees to repair or replace, without charge, any merchandise proved defective in material or workmanship.

- Warranty Period: One year after date of shipment.

      - USAGE RIGHTS

        - Owner retains copyright for unlimited reproduction of photographs and archives generated by the system.

      - MAINTENANCE

        - Maintenance Service: Provide service and maintenance of camera system for entire Construction period.

          - Examine monthly: clean and adjust equipment.

          - Provide remote emergency repair service by System Vendor seven days a week to ensure uninterrupted camera service. Provide personnel on-site to assist System Vendor as needed during working hours. Provide replacement parts and components due to system failure, damage, or theft within two business days.

          - Maintenance service shall not be assigned or transferred to another agent or subcontractor without prior written consent of Owner.

          - Require system vendor to proactively monitor the system by means of service and maintenance contract. If no connection is made within a span of time not to exceed 24 hours during regular business days, require system vendor to notify Contractor and commence troubleshooting.

            - Provide necessary staff during troubleshooting to verify power availability, to remove and replace system, and to verify functioning phone lines or Internet access for dialup and Ethernet based systems.

PART 2- PRODUCTS

  - MANUFACTURERS

- Manufacturers: Subject to compliance with requirements, provided products by the following:

  - EarthCam, Inc., 201-488-1111, http://www.EarthCam.Net.

- Substitutions: Comply with provisions of Division 1 Section “Products Requirements”.

2.02	   SYSTEM REQUIRMENTS

- The indoor/outdoor camera system shall consist of a tamper and impact resistant enclosure with integrated camera and weather resistant powder coated aluminum housing to be mount as a fixed pole, wall, parapet or non-penetrating roof mount.

- The camera shall take high-resolution 8 Megapixel digital images every 15 minutes and provide live video.

- The camera shall upload both images and video over a wireless cellular modem, a wireless point-to-point system or a RJ-45 hardwired connection to a DSL or cable modem.

- The content shall be sent to a secure, password protected website with an Interface and Online Software features provided by the Vendor as a Managed Service.

- The system will operate on 120VAC (230VAC Available) voltage and have a maximum power consumption of 130VA.

  - EQUIPMENT

- Camera: Integrated 8 Megapixel high-definition camera and lens assembly consisting of a charge coupled device (CCD) camera with a remotely controlled focal length lens with the following features:

  - Imager: 1/2.5” CCD 8 Megapixel.

  - Resolution: 3264x2488 Pixels = 8Megapixels.

  - Lens: Zoom 6mm – 72mm capable of 12x Optical, 4x Digital.

  - IP Addressing: Dynamic or Static.

  - Auto Features: Focus, ISO, shutter, and white balance.

- Camera Enclosure: Powder coated aluminum housing with stainless steel fittings for padlocks, with the following features:

  - Built in thermostatically controlled heater and blower.

  - Impact resistant viewing window, 0.25” (6.35mm) thick.

  - Two UL rated compression glands (Liquid Tight Fittings Available).

  - Window wiper to remotely clean the viewing window

  - Forward opening gas assisted lid.

  - Adjustable camera mounting sled.

  - Thermostatically controlled heater and defroster.

- Overall System:

  - Dimensions: 5.5” W x 6.25” H x 15.5” L (13.97cm W x 15.88cm H x 39.37cm)

  - Weight: 16 Lbs (7.26Kg)

- Quantity of Cameras: As required by Owner.

  - INTERFACE AND ONLINE SOFTWARE

- Remote Access: Contractor’s System Vendor shall provide an internet based interface and online software as a managed service, to allow the viewing of all high-definition digital still images captured and stored and live video, from any location with internet access via a secure password protected website.

- Maintain images on the System Vendor’s website for reference available at all times during the life of the project and for not less than 60 days after completion.

- Online Interface Features:

- Software delivered by Vendor as a Managed Service.

- Displays Companies logo and project name.

- Capable of viewing actual live video,

- Picture in Picture to view live video, while viewing high definition images.

- Calendar based navigation system for selecting specific images.

- Multifunction image browsing.

- Pan, tilt and zoom control capability within a high-definition image.

- Onscreen button for wiper control to allow remote cleaning of the viewing window.

- A multiview screen to view all of the cameras on a project at the same time.

- Graphical mark-up tools for detailing and creating overlays on images.

- Graphical weather applet displaying ten points of local weather data.

- Remote solar monitoring screen displaying the DC amperage output of the solar panels.

- Remote battery monitoring screen displaying battery voltage, temperature and status.

- Remote cellular monitoring screen displaying connectivity, network traffic and modem temperature.

- Share image tools: save, print, email and post to message board or mobile devices.

- Automated progress reports in Power Point, Open Office and PDF formats.

- Aerial and satellite photography library.

- Time lapse features include – Instant time lapse play back by day, week, month or year.

- Machine to machine self-healing technology that automates maintenance of camera up to 288 times daily.

- Account security features include – Four levels of password protection, IP address block/ permission and SSL protection of the user login password.

- EXECUTION

      - preparation

        - Unpack camera system components and save packing materials (box and foam) for future shipment of camera system including associated appurtenances and mounting equipment to Owner or Manufacturer as required.

      - INSTALLATION

        - General:

          - Install camera system in accordance with manufacturer's printed instructions, State and Municipality codes and requirements and approved submittals.

          - Install units plumb and level and at proper angle to provide maximum field of view of on-site operations.

          - Securely and rigidly anchor products in place.

          - Connect cameras to power.

        - Position camera so that field of view of approximately 51º horizontal and 39º vertical covers intended area of site.

          - Install camera at elevation that will provide uncompromised visual coverage.

          - Install camera so that position of sun or man made light sources will not come into direct contact with field of view of camera at any time during construction.

      - FIELD QUALITY CONTROL

        - Preinstallation Testing: Test camera on-site at ground level prior to mounting unit in its intended elevated position.

          - Contact System Vendor not less than 24 hours in advance of installation for testing.

          - Connect unit.

          - After 30 minutes, contact System Vendor and require System Vendor to remotely confirm camera is operating properly.

          - Install cameras in approved locations.

      - CLEANING

        - Clean installed items using methods and materials recommended in writing by manufacturer.

        - Clean camera system components, including camera-housing windows, lenses, and monitor screens.

      - instruction

        - Engage a factory-authorized service representative by phone to instruct Contractors personnel in procedures to adjust and maintain camera equipment.

          - Instruct personnel on procedures and schedules for troubleshooting and maintaining equipment.

          - Explain methods of determining optimum alignment and adjustment of components.

      - OPERATION, TERMINATION, AND REMOVAL

        - Maintenance: Maintain camera equipment in good operating condition on a 24-hour basis until removal.

        - Termination and Removal: Removal of camera system when instructed by the owner.

          - Camera system including associated appurtenances and mounting equipment are property of Owner.

END of section